Physics						Name ________________________

Mr. Draper					Worksheet

Circuits

1.               R1=2 			Find: 	RT =			PT =

12 V                     R2 =	                       	I =			P1 =

4 

V1=			P2  =

V2 =

2.    					Find:  	RT =			PT =

16 V	     R1=		R2=		IT =			P1 =

12	  24 

I1 =			P2 =

I2 =

3. Eight Christmas tree lights are connected in series to line providing 120 V.  Each light is 10 watts.

Find:  I = 		V1 =			R1 =			RT =

4. A 30  blender (R1), a 15  microwave (R2) and a 10  toaster (R3) are hooked in parallel to a 120 V line.  They are all operating.

Find: 	RT =			IT =				PT =

I1 =			P1=

I2 =			P2 =

I3 =			P3 =

5. In the circuit in problem # 4, the blender was turned off.

Find: 	RT =			IT =				PT =

I2 =			P2=

I3 =			P3 =

6. In the above circuit, how much heat was produced by the toaster in 3 minutes?

7.            						Find:	RT=		I1 =

R1= 31

110 V								IT =		I2 =

R2 =		   R3 =		V1=		I3 =

60		    10

V2=		I4=

R4=

30	V3=

V4 =

8. The damage caused by electric shock depends on the current flowing through the body.  A 1 milliAmp current can be felt.  Five mA is painful.  Above 15 mA a person loses muscle control and 70 mA can be fatal.  A person with dry skin has a resistance from one arm to the other of about 105 ohms.  When the skin is wet, the resistance drops to about

5 x 103 ohms.

a) What is the minimum voltage placed across the arms that would produce a current that could be felt by a person with dry skin?

b) What current would the same voltage produce if the person had wet skin?

9. Find the reading of each ammeter and each voltmeter:

V1=		V2=		V3=		I1=		I2 =		I3=

10. An electric motor operates a pump that irrigates a farmer’s field by pumping

10,000 kg of water a vertical distance of 8 m each hour.  He motor has an operating resistance of  22 ohms and is connected across a 110 volt source.

a) What is the current in the motor?

b) How efficient is the motor?